CRC Block

Cyclic Redundancy Check

CONTENT

CRC Hardware Block Specification	2

1. Overview	2

2. Block Diagram	2

3. Interface Specification	3

4. Functional Description	4

4.1 CRC Calculation	4

4.2 CRC Polynomial Options	4

4.3 CRC Calculation Algorithm	4

4.4 Output Handling	5

5. Operational Modes	6

5.1 Single Calculation Mode	6

5.2 Continuous Calculation Mode	6

6. Timing Specifications	6

CRC Hardware Block Specification

1. Overview

This specification defines a Cyclic Redundancy Check (CRC) hardware block capable of computing CRC checksums for data with a width of 32 bits. The block supports multiple CRC polynomial options: CRC-3, CRC-8, CRC-16, and CRC-32.

2. Block Diagram

3. Interface Specification

| Signal Name | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | System clock – 120MHz |
| rst_n | Input | 1 | Active low reset |
| data_in | Input | 32 | Input data for CRC calculation |
| data_valid | Input | 1 | Indicates valid data is present on data_in |
| crc_select | Input | 2 | Selects CRC polynomial (00:CRC-3, 01:CRC-8, 10:CRC-16, 11:CRC-32) |
| crc_init | Input | 32 | Initial value for CRC calculation |
| crc_out | Output | 32 | CRC calculation result (width depends on selected CRC) |
| crc_valid | Output | 1 | Indicates valid CRC result is available |
| busy | Output | 1 | Indicates the block is processing |

4. Functional Description

4.1 CRC Calculation

The CRC block calculates a checksum based on the input data and the selected CRC polynomial. The calculation follows the standard CRC algorithm with the following steps:

- Initialize the CRC register with the value from crc_init

- Process the 32-bit input data

- Output the final CRC value

4.2 CRC Polynomial Options

| CRC Type | Width | Standard Polynomial | Hexadecimal Representation |
| --- | --- | --- | --- |
| CRC-3 | 3 | x³ + x + 1 | 0x3 |
| CRC-8 | 8 | x⁸ + x² + x + 1 | 0x107 |
| CRC-16 | 16 | x¹⁶ + x¹⁵ + x² + 1 | 0x18005 |
| CRC-32 | 32 | x³² + x²⁶ + x²³ + x²² + x¹⁶ + x¹² + x¹¹ + x¹⁰ + x⁸ + x⁷ + x⁵ + x⁴ + x² + x + 1 | 0x104C11DB7 |

4.3 CRC Calculation Algorithm

4.3.1 Mathematical Foundation

A CRC is calculated by treating the input data as a polynomial over GF(2) (binary Galois Field) and performing polynomial division by the CRC generator polynomial. The remainder of this division is the CRC value.

4.3.2 Detailed Algorithm

For a 32-bit data input and the selected CRC polynomial, the calculation proceeds as follows:

(This guidance is for a single bit.  Have to implement it * 32 bits: )

- Initialization: Load the CRC register with the initial value (crc_init)

- Data Processing: For each bit of the 32-bit input data, starting from the most significant bit (MSB):

XOR the MSB of the current CRC register value with the current data bit

If the result is 1:

  - Shift the CRC register left by 1 bit

  - XOR the CRC register with the CRC polynomial

If the result is 0:

  - Shift the CRC register left by 1 bit

- Finalization: The final value in the CRC register is the CRC checksum

4.3.3 Hardware Implementation Approaches

The hardware block supports parallel processing implementation method:

- Bit-by-Bit Processing:

  - Process one bit at a time as described in the algorithm above

  - Simple but requires 32 clock cycles for a 32-bit input

- Parallel Processing:

  - Process multiple bits in parallel using combinational logic

  - For 32-bit data, implement a 32-stage linear feedback shift register (LFSR)

  - Achieves single-cycle CRC calculation for the entire 32-bit input

  - Most efficient for this application

4.3.4 Parallel CRC Calculation

For the 32-bit parallel implementation:

- Express the CRC calculation as a matrix multiplication in GF(2)

- Pre-compute the transformation matrix based on the CRC polynomial

- Implement the matrix multiplication as a network of XOR gates

- The result is computed in a single clock cycle

The mathematical representation for parallel CRC calculation:

CRC_next = (CRC_current << 1) ^ (MSB(CRC_current) ? CRC_polynomial : 0)

For a 32-bit parallel implementation, this operation is expanded to process all 32 bits simultaneously through a network of XOR gates derived from the polynomial.

4.4 Output Handling

- For CRC-3: Only the 3 least significant bits of crc_out are valid

- For CRC-8: Only the 8 least significant bits of crc_out are valid

- For CRC-16: Only the 16 least significant bits of crc_out are valid

- For CRC-32: All 32 bits of crc_out are valid

5. Operational Modes

5.1 Single Calculation Mode

- Set crc_select to the desired CRC type

- Set crc_init to the desired initial value

- Assert data_valid with valid data on data_in

- Wait for crc_valid to be asserted

- Read the result from crc_out

5.2 Continuous Calculation Mode

For processing multiple data blocks with the same CRC:

- Set crc_select to the desired CRC type

- Set crc_init to the desired initial value for the first block

- Assert data_valid with valid data on data_in

- Wait for crc_valid to be asserted

- Read the intermediate result from crc_out

- For subsequent blocks, set crc_init to the previous crc_out value

- Repeat steps 3-6 for each data block

6. Timing Specifications

| Parameter | Description | Value |
| --- | --- | --- |
| t_setup | Setup time for all inputs before clock edge | 2 ns |
| t_hold | Hold time for all inputs after clock edge | 1 ns |
| t_clk_to_out | Clock to output delay | 3 ns |
| t_calculation | Maximum calculation time | 2 cycles |